Session Planning Template

| Name & Date | Liong Lih Bing |
| --- | --- |
| Program Title | Hearing Conservation – Protect It Before You Lose It! |
| Learning Outcome | - Describe at least two effects of excessive noise on hearing and recognize early warning signs - State the key >82 dB(A) and >85 dB(A) workplace triggers under the Noise Exposure Regulations 2019 - Explain why audiometric testing is required, who must attend, when it occurs and how to prepare - Demonstrate correct fitting, use, fit-checking and care of the assigned personal hearing protector - Choose the correct action in common noise-exposure scenarios |

| Sequence | Topics/Content | Training Methods | Training Resources | Time Timeframe |
| --- | --- | --- | --- | --- |
| Intro | Welcome, relevance and learning outcomes - Opening vote: “Which route lets noise reach your inner ear?” - Ground rule: protect first, ask early when unsure | - Facilitated show-of-hands vote - Brief pair discussion and trainer debrief | - PPT slides - Company-issued hearing protector as a visual prop | 3 min |
| Body | Noise dose and the 2019 legal triggers - Dose = noise level x exposure time; louder noise uses the safe dose faster - Excessive noise: daily exposure >82 dB(A) or dose >50% → information, annual training and supervision - Exposure limit: daily >85 dB(A) or dose >100%; maximum >115 dB(A); peak >140 dB(C) → controls, hearing-protection zones and audiometry | - Questioning using site examples - Threshold comparison and quick call-out: “82 or 85?” | - PPT slides - Latest site noise risk assessment/noise map | 5 min |
|  | How excessive noise affects hearing - Sound travels through the outer and middle ear to the cochlea; repeated high noise damages sensory hair cells, which do not regrow - Effects: temporary threshold shift, permanent noise-induced hearing loss, tinnitus, muffled speech and reduced ability to hear warnings/communication - Early warning signs: ringing after work, turning volume up, difficulty understanding speech – especially in background noise | - Visual explanation and guided questions - “Spot the sign” call-out using familiar work situations | - PPT slides - Ear-pathway and hair-cell visuals - Site incident with names removed | 5 min |
|  | Control the noise and choose suitable protection - Use the hierarchy: eliminate/substitute engineering controls administrative controls personal hearing protection - Select the approved protector for the assessed noise, task, hygiene needs and compatibility with other PPE; double protection only where required | - Mini case: identify the strongest practicable control - Compare plug vs earmuff options for production, maintenance, WWTP and QA/QC tasks | - PPT slides - Site control plan and noise-zone signage - Samples of approved earplugs and earmuffs | 4 min |
|  | Foam earplug demonstration and practice - ROLL the entire plug into a thin cylinder; PULL the ear up and back; INSERT deeply; HOLD 20-30 seconds while it expands - Fit-check by cupping/un-cupping hands over the ears; sound should change very little when the seal is good | - Trainer demonstration - Participant guided practice with buddy observation - Corrective coaching using the roll-pull-hold checklist | - PPT slides - Clean disposable foam plugs (one pair/person) - Hand sanitizers/tissues | 4 mins |
|  | Reusable plugs, earmuffs and continuous wear - Reusable plug: clean hands, pull ear up/back, insert until all flanges seal; clean and store as instructed - Earmuff: fully enclose ears; keep cushions clean; remove hair/cap straps from the seal; inspect cushions, cups and headband tension - Put protection on before entering the zone and wear it for the entire exposure; ordinary music earbuds are not a substitute | - Trainer demonstration and 30-second fit challenge - Buddy check; seal, comfort, compatibility and condition - Show an intentionally damaged sample and ask: “Use or replace?” | - PPT slides - Reusable plugs and earmuffs - Damaged/dirty protector samples and inspection checklist | 6 min |
|  | Audiometric testing: purpose, timing and preparation - Purpose: establish a baseline, track change and trigger early action – not pass/fail or punishment - For workers exposed above the noise-exposure limit: baseline within 3 months of starting exposure; annual testing at no cost through an approved audiometric testing centre - Before the test: 14 hours of quiet below 80 dB(A); a protector cannot replace quiet. Employer informs results within 21 days; temporary threshold shift is re-tested within 3 months; abnormal audiograms follow the OHD route | - Explain the baseline annual test result/follow-up pathway - Teach-back: participants state two pre-test actions | - PPT slides - Site audiometry notice/appointment process - Approved ATC details and company HCP procedure | 5 min |
| Summary | Review, key message and commitment - Myth or fact: hearing loss is always noticeable; protection may be removed briefly; audiometry is only a pass/fail test - CONTROL the source. WEAR protection correctly. TEST hearing regularly. REPORT early. - Each participant states one action for the next shift | - Myth/fact movement - One-sentence commitment round | - PPT slides - Commitment pledge via Mentimeter | 6 min` |
|  | Knowledge Check and Output Assessment - Multiple choice questions with workplace scenarios - Success criteria: at least 80% correct plus satisfactory observed fit-fit-check | - Individual answers using handphone - Trainer observation of practical fitting skill | - PPT slides – QR codes - Practical fit observation checklist | 20 mins |